| Arc Welding |  |  |  |  |  | SOP Number: | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 046SOP | 041RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Welding flash - eye injury; Burns; Electric Shock. |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Ensure the welder is used in a suitable safe work area. - Ensure all cables are correctly fastened, free of cracks and splits and unwound before work. - Keep the area clean & free of grease, oils & flammables. - Ensure other workers are protected from any UV flash. - Ensure the area is well ventilated (with fume extraction as required). |  |  |  |  |  |  |  |
| OPERATIONAL SAFETY CHECKS - Ensure the welder is correctly set up for current, voltage and good earth. - Ensure that other workers in this locality are protected from any UV & IR radiation flash. Always close the UV curtain to the welding bay or erect a UV screen. - Ensure the welding return cable (earth) makes firm contact to provide a good electrical contact. - Ensure your work-piece has been prepared to be free of any paint, oxides or other surface finishes ensuring a good electrical contact. - Take particular care to avoid accidental UV welding flash to the skin or eyes. - Never leave the welder running unattended. - Regularly inspect the welding tip and shield for damage. - When welding is finished or interrupted, turn off the welder and secure the handpiece safely. - Ensure the work area can isolate hot areas from workers to avoid hot surfaces until job is cool. |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Ensure the welder is switched off (and fume extraction if used). - Check for any damage to welder and cables at the end of job. Wind up cables. - Hang up the welding handpiece and cables securely. - Check contact points for damage or corrosion. - Leave the work area and welding bench in a safe, clean, and tidy condition. |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.