| Belt and Disc Sander |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 052SOP |  | 048RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Abrasions; Burns to Skin; Eye Injuries; Excessive Dust; Flying Objects, Electric Shock; |  |  |  |  |  |  |  |  |
|  |  |  | Rings and jewellery must not be worn. |  | Gloves must not be worn when using this machine. |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check workspaces and walkways to ensure no slip/trip hazards are present. - Ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted). - Check the table is set not more than 2mm from disc. - Check belts and discs are in a serviceable condition. - Operator must be positioned out of direct line of abrasive belt at all times. - Ensure dust extraction is on before operating machine. - Faulty equipment must not be used. Immediately report suspect machinery. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Allow machine to reach maximum revolutions before operating to avoid overloading. - Always place material on the table on the downward side of the disc travel to hold it down on the table surface. - Hold material firmly against stops or table before applying pressure on abrasive. - Keep fingers clear of disc or belt while sanding. - Never leave the machine while it is running. - Before making adjustments switch off and bring the machine to a complete standstill. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Switch off the machine. - Leave the machine in a safe, clean and tidy state. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.