| Blower Vacuum - Petrol |  |  |  |  |  | SOP Number: | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 030SOP | 005RA |
| DO NOT use / operate this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Mechanical, Hazardous Chemicals, Hazardous Manual Tasks, Gravity, Noise, Fire, Extreme Temperatures |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Jewellery, loose clothing and hair to be secured so that it cannot be drawn into the air inlet. - Check machine for loose/missing nuts, bolts and screws. Tighten and/or replace as needed. - Inspect the parts of the mulching cutting head (where fitted). - Ensure the debris bag (where fitted) is in sound condition and is properly fitted with no gaps. - Ensure all guards are fitted, secure and functional. - Faulty equipment must not be used. Immediately report suspect machinery. - Ensure all other persons are clear of the immediate work area. - Fuel the blower when cool and before starting work. |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Ensure no person or animal is endangered when operating equipment. - Adjust the shoulder harness as required to ensure it is in the most comfortable position. - Keep a firm grip. Hold handles with fingers together encircling handles. - Maintain a straight wrist position. Avoid using your wrist in a bent, extended or twisted position. - Maintain footing and balance at all times. DO NOT stand on slippery, uneven or unstable surfaces. DO NOT work in odd positions or on ladders. - Select the mode indicator to adjust the nozzle position for blow or vacuum. - DO NOT direct the blower in the direction of other people. - Take wind conditions into account. Avoid open doors and windows. - Minimise dust by using blower at lower speeds. - If the blower vacuum starts to vibrate abnormally, switch off and check immediately. - Allow the engine and muffler to completely cool before refuelling or performing any maintenance. - Switch off before removing or replacing the debris bag (where fitted). - DO NOT place hands in bag when emptying. Sharp debris can cause injury. |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Turn off fuel supply when work completed. - Remove any foreign material from motor air inlet and nozzle - Allow blower to fully cool before putting away. |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.