| Chainsaw - Petrol |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 067SOP |  | 069RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Cuts and lacerations / Hearing loss / Flying debris / Eye injuries / Entanglement / Crush injuries / Death |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Ensure the chainsaw is clean. - Check for loose, worn or damaged parts. (Do not operate till repaired / replaced) - Inspect fuel lines, carburettor for leaks. (Do not operate till repaired / replaced) - Check effectiveness of chain brake and operating controls. - Ensure safety guards and other safety features are secure and functioning) - Ensure chain is sharp and correctly tensioned. - Ensure you are in good physical condition to operate chainsaw. - Ensure PPE is available and fits correctly. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Follow all safety precautions in the operator’s manual – improper use can cause serious or fatal injuries. - Ensure area is clear of obstructions / other personnel. - Start chainsaw resting on the ground. - Maintain balance and secure footing when operating. - Start cut with chainsaw at full speed and bumper is in contact with work piece. - Be aware of the guide bar nose at all times when chain is in motion. - Pay full attention at all times and be alert for movement of the work piece. - Apply chain brake when chainsaw is at rest. - Never use chainsaw over or above shoulder height. - Allow chainsaw to cool before refuelling. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Clean chainsaw at completion of task. - Sharpen saw chain as per manufacturer’s requirements. - Service chainsaw as per manufacturer’s recommendations. - Keep work area clean and tidy. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.