| Compound Mitre Saw |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 049SOP |  | 045RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Rotating & Moving Parts; Pinch & Squash Injuries; Flying Chips and Debris; Eye Injuries |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Ensure the saw is properly secured to a workbench by bolts/ clamps at approximately hip height. - Ensure the saw is operated on an RCD protected circuit. - Check workspaces and walkways to ensure no slip/trip-hazards are present. - Check that all safety guards are in position and are operational. - Ensure you are familiar with the operation of the ON/OFF DOL switch and Emergency stop. - Keep table and work area clear of all tools, off-cut timber and sawdust. - Start the dust extraction unit before using the machine. - Faulty equipment must not be used. Immediately report suspect equipment. - Complete On Guard Safety training and submit to teacher. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - The maximum cut for the machine must not be exceeded. - Ensure all adjustments are secure before making a cut. - Use clamps to secure and support the workpiece to a stable platform. Do not use a length stop on the free scrap end of a clamped workpiece. - Do not cut more than one workpiece at a time. - Before turning on the saw, perform a dry run of the cutting operation to ensure that no problems will occur when the cut is made. - Do not start the saw with the blade touching the workpiece. Allow the blade to reach full speed before contacting the workpiece. - Avoid reaching over the saw line. Do not cross arms when cutting. - Always use the right hand to pull the saw down, keep the left hand, especially the thumb well clear of the line of cut. - If workpiece is bowed or warped, clamp it with the outside bowed face toward the fence. - After finishing the cut, release the switch, hold the saw arm down and wait for blade to stop before removing work or off-cut piece. - Disconnect the plug from the power source and bring the machine to a complete standstill before making any adjustments. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Switch off and lock out the machine. Leave the machine in a safe, clean and tidy stat |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |