| Cut Off Saw |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 051SOP |  | 044RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Moving, rotating and sharp parts; Dust and fumes; Electricity Pinch and crush; Eye injuries; Noise; Burns and hot ejected waste material |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Ensure the power tool has a suitable safe work area. - If changing cut-off wheel, check for appropriateness and arbor hole size. - Inspect the cut-off wheel for chips and cracks. - Check that the guards are all in place and in good working order. - If fitted with a depth stop, ensure that it is in proper adjustment/alignment. - Familiarise yourself with the operation of the ON/OFF switch. - Ensure your work area is clear of slip/trip hazards and flammable materials. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Ensure all adjustments to the machine are secure before making a cut. - Do not hold your work piece by hand. It may become very hot while being cut. - Use the vice to clamp the work and properly support any overhanging portion so the work piece is level with the base of the machine. - Do not use a length stop on the free off-cut end of a clamped work piece. - Allow the machine to reach full speed before contacting the work piece. - Do not have any part of your body in line with the path of the abrasive disc. - Ease the abrasive disc against the work piece when starting. Do not force tool. - Keep hands away from cutting. Do not attempt to remove cut material while the abrasive wheel is moving. - After finishing the cut, release the switch, hold the saw arm down and wait for the disc to stop before removing your work piece or off-cut piece. - Before making any machinery adjustments, bring the machine to a complete standstill, and then disconnect the AC plug from the power source. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Ensure ventilation openings and switches are kept clean and free of foreign matter. - Leave the work area in a safe, clean and tidy condition. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |