| Electric Sander - Handheld |  |  |  |  |  | SOP Number: | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 071SOP | 074RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Cuts and abrasions / Exposure to Dust / Exposure to projectiles / Moving parts and electrical hazard |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Task (e.g. Drawings, instructions, specifications etc.) is clearly understood. 2. Ensure work piece is securely clamped. 3. Identify location of ON / OFF switch. 4. Faulty equipment must not be used. 5. Ensure dust extractor bag is empty and attached. 6. Ensure Power supply is off when connecting or disconnecting. |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Do not leave running machinery unattended. - Hold orbital sander securely. - Secure work where possible. - Use light pressure to allow machine to cut freely. - Allow orbital sander to stop before lifting from work. |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Return sander to storage area. - Ensure good housekeeping practices are in place to minimise waste build-up. - Sweep down bench and tidy work area. |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.