| FORKLIFTS |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 084SOP |  | 090RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Tip overs, Collisions, Overloading, Pedestrians, Unstable ground, Overhead Obstructions, Combustibility due to gases, Poorly maintained forklift / attachments. |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Locate and ensure you are familiar with all machine operations and controls. - Check brakes, lights and horn before use. - Ensure reversing beeper and warning lights are operational. - Ensure seat belt/safety restraint is in good condition. - Know the capacity of the forklift before using it. Do not use a forklift without a load rating plate. - Check gas-powered vehicles for gas leaks in fuel lines. - Ensure the lifting forks are in sound condition and centred either side of the mast. - Check tyre pressures. Never drive with a flat or under-inflated tyre. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Watch out for pedestrians. - Ensure the lifting tines are secure into the pallet and the load is stable before lifting or driving off. - Be careful of ceiling clearance or overhead obstructions when raising the mast. - Always remember that the safe working load of a forklift reduces as the mast is tilted forward. - Always put the heavy end of the load against the load backrest. - When approaching a blind corner, use horn and drive slowly. - Always have someone guide you if a load restricts your vision. - Slow down when changing direction or on wet or greasy surfaces. - Avoid harsh braking, especially when carrying a load. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - When stopping the forklift: - Park on even ground and lower the forks to the ground. - Shift the gear selector to park position and apply the parking brake. - Turn off the ignition and remove the keys. - For gas-powered vehicles, turn off gas and don’t park near an ignition source, doorway or pit. REFUELLING - For gas-powered vehicles, change gas bottles in a well-ventilated area clear of a naked flame or source of ignition. - For battery-operated vehicles, charge in well-ventilated area. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.