| Generator |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 055SOP |  | 052RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Rotating and moving parts; Electric Shock; Noise; Exhaust fumes Burns |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY Two people as a minimum are to lift the generator Ensure generator is on a firm and level surface at a short distance from the work area Ensure generator is adequately earthed Wear eye protection and gloves when refuelling Ensure generator output is sufficient to power the necessary equipment or electrical circuit Check to make sure that generator has been certified and tagged by an electrician that the certification is current Check residual current device (RCD) and overload breaker switches are operational Where an RCD is not fitted, a portable RCD must be used |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS When starting adopt a tripod stance, do not twist your body and do not pull cord past your body Only use generators outdoors in well ventilated areas Ensure no smoking, naked flames or ignition source Carry fuel in approved clearly marked containers only Ensure fuel cap secure Use funnels and/or spout and take care not to spill on hot parts Allow generator to cool down before refuelling |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING 1. Switch off and the machine. Leave the machine in a safe, clean and tidy state |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |