| Grinder (Bench) |  |  |  |  |  | SOP Number: | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 043SOP | 088RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Rotating and sharp parts; Electricity; Noise; Pinch and squash Dust and fumes; Eye injuries; Burns and hot ejected waste material |  |  |  |  |  |  |  |
|  |  |  |  |  | Rings & jewelly must not be worn |  |  |
| # PRE-OPERATIONAL SAFETY - Check workspaces and walkways to ensure no slip/trip hazards are present. - Ensure all guards and safety shields are in position before starting the grinder. - Ensure that the wheels do not touch the work rest and that the gap between wheel and rest is no greater than 1.5mm. - Check that wheels are running true and are not glazed or loaded. - Locate and ensure you are familiar with the operation of the ON/OFF starter. - Faulty equipment must not be used. Immediately report any suspect machinery. |  |  |  |  |  |  |  |
| OPERATIONAL SAFETY CHECKS - Stand to the side of the wheels when starting up. - Let the wheels gain maximum speed before starting to grind. - Do not grind on the side of the wheel. - Small objects must not be held by hand. - Never leave the machine running unattended. - Do not bend down near the machine whilst it is running. - Never force the work-piece against a wheel. - Slowly move the work-piece across the face of the wheel in a uniform manner. - Coolant spilt on the floor should be immediately absorbed. |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Switch off the grinder. - Leave the machine in a safe, clean and tidy state. |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.