| Grinder (Cordless) |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 050SOP |  | 046RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Rotating and sharp parts; Electricity; Noise; Pinch and squash Dust and fumes; Eye injuries; Burns and hot ejected waste material |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY 1. Is your work area suitable and allow for the safe use of the angle grinder without interfering with other students/activities? 2. Is your work area clear of trip hazards and flammable materials? 3. If changing a grinding disc, check for appropriateness and arbor hole size. 4. Have you inspected the grinding disc for chips and cracks? 5. Are all guards in place and in good working order? 6. Does the grinding disc guarding cover half of the rotating disc? |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS 1. Do not attach the battery until adjustments have been made. 2. Do not fit a grinding disc that has been dropped, damaged or become wet at any stage. 3. Ensure your work piece is firmly secured and supported. 4. Keep fingers and hands clear of the rotating disc. 5. Use the grinder only in a designated work area – preferably behind a portable screen. 6. Be aware of flying hot sparks. Hold the grinder so that any sparks fly away from you and anyone nearby, and away from all flammable materials. 7. Allow the grinder to reach operating speed, then apply load gradually. Maintain a proper and steady footing at all times. 8. Maintain complete control. Always operate with both hands. Maintain a proper and steady footing at all times. 9. Do not apply excessive force and avoid prolonged use. 10. Grinder will not operate if the battery charge is too low. 11. Violent kickback can occur, particularly when cutting. 12. Leather gloves must be worn when handling hot metal. 13. Turn off after use. Do not place the grinder down until the disc has stopped rotating. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING 1. Leave the work bench and machine in a safe, clean and tidy condition. 2. Return the grinder to storage and remove the battery for recharging. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |