| Hedge Trimmer - Electric |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 028SOP |  | 053RA |
| DO NOT use / operate this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Mechanical, Hazardous Chemicals, Hazardous Manual Tasks, Gravity, Noise |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check for obvious faults and defects including electric cord (if not battery powered) for signs of wear or damage. - Equipment has current test and tag (if not battery powered). - Identify ON/OFF switch/s. - Wear appropriate PPE. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Transport the machine with the scabbard on and the blade facing away from you. - Hold trimmer firmly with both hands. - Ensure firm footing. - If excessive dust is present use an appropriate respirator. - Move your body with the trimmer; do not twist from the waist. - Keep cord free from cutting blades at all times (if fitted) - DO NOT cut over shoulder height. - DO NOT walk with trimmer whilst blades are moving - DO NOT use in the rain or wet conditions. - Ensure bystanders are at least 5 metres away from your working position. - If using the machine for prolonged periods take breaks to avoid tiredness and fatigue. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Ensure cutters have stopped before removing waste material - Ensure cutters have stopped before placing trimmer on the ground. - Clean the cutting blade with a resin solvent. - Ensure equipment is returned to storage after use and safely stored. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.