| Hedge Trimmer - Petrol |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 027SOP |  | 054RA |
| DO NOT use / operate this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Mechanical, Hazardous Chemicals, Hazardous Manual Tasks, Gravity, Noise, Fire, Extreme Temperatures |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check for obvious faults and defects. - If fuel leaks are identified do not start the machine until the leak is fixed. - Identify ON/OFF switch. - Cutting blades have a resin solvent sprayed on them for lubrication. - Adjust any harness or handles to suit your height and reach. - Wear adequate and appropriate PPE. - Inspect hedge for any obstacles that may become entangled in the cutting blade. - Check fuel level and refuel in well ventilated areas. - Wipe up any spilt fuel. - If fuel is spilt on clothing, change before use. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - DO NOT drop start the machine, place it on the ground to start. - Only operate the machine in the open to avoid effects of the exhaust fumes. - Transport the machine with the scabbard on and the blade facing away from you. - Hold trimmer firmly with both hands and ensure firm footing. - If excessive dust is present use an appropriate respirator. - Move your body with the trimmer; do not twist from the waist. - DO NOT cut over shoulder height. - DO NOT walk with trimmer whilst cutter is spinning. - DO NOT use in the rain or wet conditions. - Ensure bystanders are at least 5 metres away from your working position. - If using the machine for prolonged periods take breaks to avoid tiredness and fatigue. - DO NOT touch the gearbox housing to avoid burns. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Ensure cutters have stopped before removing waste material - Ensure cutters have stopped before placing trimmer on the ground. - Clean the cutting blade with a resin type solvent. - Ensure equipment is returned to storage area after use and stored safely. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.