| Lawnmower - Petrol |  |  |  | SOP Number: | RA Number: |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 024SOP | 113RA |
| DO NOT use / operate this equipment or complete the task unless you have been trained. |  |  |  |  |  |
| POTENTIAL RISKS: Mechanical, Hazardous Chemicals, Hazardous Manual Tasks, Gravity, Noise, Fire, Extreme Temperatures |  |  |  |  |  |
|  |  |  |  | Put in Picture of equipment / plant |  |
| # PRE-OPERATIONAL SAFETY - Check unit for loose/missing nuts/parts. - Inspect fuel lines and tank for leaks. - Ensure that all guards are fitted and secure. - Check blades for damage and cracks. - Faulty equipment must not be used. Tag out and report suspect machinery immediately. - Check area to be mowed for any objects that could be ejected by the mower or damage the mower. |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - DO NOT use in close proximity to other workers. - Clear the area to be mowed of debris that may be thrown. - When you start up the machine, listen for unusual noises or vibrations. - Keep your hands and feet away from moving parts at all times. - Watch for ejecting material. Ensure no person or animal in endangered when operating equipment. - Avoid mowing in wet conditions due to lack of traction. - Maintain a proper balance and secure footing. - Operate at a speed slow enough to keep control over unexpected hazards. - Mow in a forward direction. - When mowing on a slope, mow across the slope. This keeps a better distance between you and the mower in case of a slip. - Allow the mower to cool down before refuelling. - Ensure good housekeeping practices are in place to minimise waste build-up |  |  |  |  |  |
| # HOUSEKEEPING - Turn off fuel supply when work completed. - Before adjusting or clearing clogs, shut off the mower, remove spark plug lead and allow it to come to a complete stop and cool down. - Remove any foreign material from in and around engine and catcher parts. |  |  |  |  |  |
| Date of last review: |  |  | Signature: |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.