| Mig Welding |  |  |  |  |  | SOP Number: | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 047SOP | 043RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Welding flash - eye injury; Burns; Electric Shock; fire; fumes |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check workspace, walkways and work leads to ensure that no slip/trip hazards are present. - Check work area is clean and free of grease, oil and flammable material. - Ensure the equipment and work area is dry to avoid electric shocks. - Check gloves, welding gun and work leads are in good condition. - Ensure the fume extractor unit is on before welding commences. - Close the UV curtain or erect safety screens to ensure the protection of others. |  |  |  |  |  |  |  |
| OPERATIONAL SAFETY CHECKS - Ensure machine is correctly set up for current, voltage, wire feed and gas flow. - Ensure work return cables make firm contact to provide a good electrical connection. - Ensure the work piece has been prepared to be free of any paint, oxides, or other surface finishes ensuring a good electrical contact. - Close curtains and ensure all bystanders are wearing proper PPE - Never leave the welder running unattended. - Avoid welding flashes. Erect UV safety curtains as required. - Regularly inspect the welding tip and shield for damage. - When welding is finished or interrupted, turn off the shielding gas at the regulator, turn off the machine and secure the handpiece. |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Switch off the machine and fume extraction. - Close gas cylinder valve. - Hang up the welding gun and welding cables. - Leave the work area in a safe, clean and tidy state. |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.