| Mortising Machine |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 061SOP |  | 062RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Moving, rotating and sharp parts; Eye injuries, Electricity, Dust Burns (friction); Ejected waste; Pinch and crush |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY 1. Ensure this power tool has a suitable safe work area. 2. Check the condition of the auger and chisel tool bit. 3. Check the operation of all fences, clamps and guards. 4. Examine the power lead and plug for obvious damage. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS 1. Always select the correct hollow chisel and auger set for the mortise to drill. 2. Tighten the chuck uniformly when the auger bit is inserted – using the correct key. 3. Always remove the chuck key before starting the mortising machine. 4. Keep clear of moving parts. 5. Never attempt to drill material that does not lie flat on the table. Irregular shaped work should be securely held in a jig, vice or clamp. 6. Set the slot in the side of the chisel bit to align with the lateral direction of your intended mortise. 7. The work piece should be moved so that the chisel is releasing chips into the already cut part of the work piece. 8. Allow the auger bit to cut at its own speed without applying excessive pressure. 9. Keep hands clear of the auger and chisel when the machine is running. 10. Do not remove waste material from the table while the machine is running. 11. Never leave the machine running unattended. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING 1. Ensure all switches, guards and clamping are cleaned. 2. Leave the work area in a safe, clean and tidy condition. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |