| Pan Brake |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 064SOP |  | 065RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Sharp Edges and Burrs / Impact from Counterweight / Squash/crush and pinch points |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Guards or safety devices shall never be removed or adjusted, except by an authorized person for maintenance purposes. - Working parts should be well lubricated and the jaws and fingers free from rust and dirt. - Ensure no slip/trip hazards are present in workspaces and walkways. - Be aware of other personnel in the immediate vicinity and ensure the area is clear before using equipment. - Familiarise yourself with and check all machine operations and controls. - Faulty equipment must not be used. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Only one operator using the machine – school staff member to assist if required - Never use pan brakes for bending metal that is beyond the machine’s capacity with respect to thickness, shape, or type. - Never attempt to bend rod, wire, strap, or spring steel sheets in a pan brake. - Remove the pan brake fingers that are in the way - use only the pan brake fingers required to make the bend. - Ensure the pan brake fingers that are not removed for an operation are securely seated and firmly tightened before the machine is used. - Ensure fingers and limbs are clear before operating the pan brake. - Lower finger clamps to work - do not drop. - Check work piece is secure & keep clear of moving counterweight (where fitted). |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Lower finger clamps to a safe position - down position resting on base plate. - Return all accessories to storage racks. - Lock-out the action of the pan brake when not in use eg a system to lock handle & bending beam from use. - Leave the work area in a safe, clean and tidy state. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.