| Polishing Machine (Buff) |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 069SOP |  | 072RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Work can be snatched if improperly presented to buff. Hair / clothing entanglement in moving machine parts / Eye injuries |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check workspaces and walkways to ensure no slip / trip hazards are present. - Locate and ensure you are familiar with the operation of the ON / OFF starter. - Both ends of the machine spindle must be covered at all times. - Use appropriate mop type and polishing compound for the task. - Faulty equipment must not be used. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Never leave the machine running unattended. - Do not bend down near machine whilst it is running. - Use the front of the wheel only. - Work only below the centre of the spindle. - Small work must be securely supported with a wooden backing. - Work should be held so that edges cannot catch. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Switch off the machine. - Leave the machine in a safe, clean and tidy state. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.