| Scaffolding - Portable |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 072SOP |  | 075RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Collapse of scaffold / Incorrect assembly / Manual handling / Struck by falling objects / Slips/trips/falls / Noise / Exposure to weather / Sprains/strains / Electrocution |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Ensure all components are well-maintained. Do not use components that are in poor condition. - Do not mix components from differently branded scaffolding systems. - Read and follow the manufacturer’s specifications, assembly instructions and warning labels. - Inspect scaffold before its first use and at the start of each day. Ensure scaffold remains level and plumb at all times - Use barricades, danger tags or warning signs to prevent unauthorised access. - Ensure edge protection (hand rails, mid-rails and toe boards) is provided. - Wear appropriate personal protective equipment. - Do not work during a storm, wet conditions or high winds. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Castor brakes are to be applied at all times while the mobile scaffold is stationary. - Access is to be only by way of an internal ladder. No climbing is allowed on the scaffold itself. - Only work on fully planked work platforms. Ensure that working platforms are kept clear of debris and obstructions along their length. Do not leave tools and equipment on scaffolds. - Scaffold is not to be moved while workers are on the scaffold work platform. - Limit the number of workers on a scaffold at any one time. - Do not exceed the safe working load of the scaffold. - Do not make any unauthorised alterations to the scaffold. - Do not permit workers or other persons to stand directly below a scaffold. - Do not drop materials from the platforms. - Do not fix high voltage leads and cables to the scaffolding. - Never use ladders or makeshift devices on top of scaffolds to increase height. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Check and maintain all scaffold parts. Repair or discard and replace any damaged pieces immediately. - Lubricate moving parts of all fittings. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.