| Sewing Machine |  |  |  |  | SOP Number: | RA Number: |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 073SOP | 076RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |
| POTENTIAL RISKS: Sharp Edges / Cuts and punctures / Eye Strain |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Thoroughly review and understand information provided in the owner’s manual. - Before using, always inspect for damage or disrepair, including power cord. - Assure ventilation openings are clear of lint and scrap cloth. - Inspect all machine settings and adjustments and modify as necessary for sewing task. - Inspect the needle to make sure it’s not bent, dulled, or damaged. Never use a bent needle. - If machine is damaged, do not use, until repairs have been made. |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Only use a sewing machine for its intended purpose as described in the operator’s manual. - Use the proper type and size needle for the fabric being sewn. - For zig-zag or any other special stitching, use a throat plate that accommodates a wide stitch. - Always maintain a safe zone, of about one inch, where fingers never enter when in operation. - Learn to start and run machine slowly and evenly. - Never sew across pins. Carefully use straight pins and when done, place in proper storage containers. - Never look away from the machine while it is sewing. If you need to look away, stop sewing first. |  |  |  |  |  |  |
| # HOUSEKEEPING - Always turn the sewing machine off and unplug from the power source after use. - Regularly clean the sewing machine with a lint brush after use. - Clean up the work space when finished with your sewing task. |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.