| Table Router |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 075SOP |  | 078RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Material kickback / Flying chips and debris / Eye injuries / Excessive noise / Contact with cutter at point of operation and during cleaning and adjustment |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check workspaces and walkways to ensure no slip / trip hazards are present. - Locate and ensure you are familiar with the operation of the ON / OFF starter and E-Stop (if fitted). - Use only materials free from defects. - Ensure cutter size conforms to machine specification. - Machine must be isolated while cutters are being installed. - Ensure all cutters are sharp and free of resin build up. - Check cutter for clearance of rotation before starting machine. - Adjust fence, guards and extraction for maximum protection and efficiency. - Do not store tools and parts on top of the machine. - Faulty equipment must not be used. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Never leave the machine running unattended. - Before making adjustments switch off and bring the machine to a complete standstill. - Feeding must be against the direction of rotation. - Deep cuts should not be attempted. - Feed work slowly into the cutter. - Jigs, fixtures and templates should be used wherever possible. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Switch off the machine. - Leave the machine in a safe, clean and tidy state. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.