| Thicknesser |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 079SOP |  | 080RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Rotating & moving parts - timber may be flung back violently / Pinch & squash injuries / Flying chips and debris / Eye injuries / Excessive noise |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check workspaces and walkways to ensure that no slip/trip hazards are present. - Check material to be planed for defects, splits, dead knots, or contaminants. - Ensure in-feed table is clear of debris and free from gum or resin residue. - Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop. - Do not exceed maximum 2mm depth of cut. - Ensure all locks are securely tightened before operating the machine. - Start the dust extraction unit before using the machine. - Faulty equipment must not be used. Immediately report suspect machinery. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Never leave the machine running unattended. - Timber less than 300 mm in length must not be planed on this machine. - Feed timber to machine with the grain. - Feed only one piece of timber at a time. - Never look through machine feed opening when cutter block is revolving. - Never plane any timber with embedded nails, and other contaminants. - Stand to one side of material being fed through machine in case of kickback. - Before making adjustments or before clearing the in-feed or out-feed tables switch off and bring the machine to a complete standstill. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Switch off the machine. - Leave the machine in a safe, clean and tidy state. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.