| Trailer BBQ |  |  |  |  |  | SOP Number: | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 037SOP | 017RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Burns; Crushing injuries; Fire; Sprains / Strains; |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Reverse the vehicle towards the trailer using another person to act as a spotter. DO NOT stand between the vehicle and the trailer when reversing. - Use the winch to raise and lower the c up and position it over the tow ball. - Secure the chains to the vehicle - Connect the cable that powers the tail lights and blinkers on the trailer and then test that they are working - Place the jockey wheel in a horizontal position in line with the trailer arm and make sure the handle is upright, then remove the trailer chocks and disengage the trailer handbrake. Make sure to activate hydraulic trailer brake before towing begins. - Ensure there is a fire extinguishers &/or a fire blanket. |  |  |  |  |  |  |  |
| OPERATIONAL SAFETY CHECKS - Position trailer in the chosen location. (Level ground, sufficient clearance around the trailer) - Engage the trailer brakes, then use the winch to disconnect the trailer from the tow bar. - Place a traffic cone in front of the tow bar and place the chocks under the wheels. - Drop the legs at the rear of the trailer and secure them with the attached pin. If holes and pins do not line up, raise or lower the trailer using the winch until they align. - Before lighting the BBQ, carry out a gas leak test on the cylinder, hose and regulator using the soap water spray. - When cooking on the BBQ, always wear a non-flammable apron and use long arm tongs to handle food. |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Remove all litter from the area and ensure all equipment is clean. - When cleaning the BBQ, ensure that it has cooled sufficiently so that there is no risk of burns. - Disconnect and wipe the grease collection pipe. - Remove any spills from the floor mat using a cloth or a mop if one is available. - Pack up the trailer. - Return trailer to the designated area for storage. |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.