| Whipper Snipper - Petrol |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 025SOP |  | 054RA |
| DO NOT use / operate this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Fire, Mechanical, Hazardous Chemicals, Hazardous Manual Tasks, Gravity, Noise, Extreme Temperatures |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check unit for loose/missing nuts/parts. - Inspect fuel lines and tank for leaks. - Ensure that all guards are fitted and secure. - Ensure the harness is attached. - Ensure cutting tool is fitted correctly. - If using the nylon line cutting head ensure line is correct dimensions. - Faulty equipment must not be used. Tag out and report suspect machinery immediately. - Keep any person or animal a minimum 15 metre radius away from the working area. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Adjust the handle/s to a position for comfort and good balance. - When starting, ensure the line or blade cannot contact the ground or any other obstruction. - The muffler side of the engine should be away from your body to avoid burns. - Maintain footing and balance at all times. DO NOT work on slippery, uneven or unstable surfaces. - Maintain a straight wrist position Avoid using your wrist in a bent, extended or twisted position for extended periods. - Always keep both hands on the control handles. DO NOT operate one-handed. - DO NOT raise the line or blade head above knee height. - Keep machine clear of fences, wires, posts, rocks, etc. to prevent kick out and cutter head damage. - Shut down immediately if the unit starts to shake or vibrate. - After shutting down the engine, keep fingers and feet away from the cutting line or blade until all rotation stops. - Disconnect spark plug wire before you work on the unit or leave it unattended. - Take extreme caution when refuelling to avoid igniting fuel on hot exhaust or engine. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Remove any foreign material from engine, cutting tool and guards, once cooled down. - Keep the work area and implement shed in a clean and tidy condition. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.